SIH26126 — GPS-Denied UGV Navigation

Isaac Sim 6.0.1 + ROS 2 Jazzy + Nav2 — Complete System Report

Workspace: ~/sih_ws  ·  Package: src/sih_isaac  ·  Commit 54c211f

27 September 2026 — RIG, KCT

An operator hands the rover a UTM Zone 44N easting/northing. The rover plans over a georeferenced GeoTIFF prior, drives 123.5 m across unstructured terrain — up an 8° slope, through a lit tunnel bored in a 3.2 m berm wall, past drums, logs, boulders, trees and walking humans — and stops 0.23 m from the commanded coordinate. No GNSS receiver, no NavSatFix, no gps_* topic exists anywhere in the system.

# Contents

1.  Measured results (committed run, seed 42)

2.  Running it yourself — the three terminals

3.  System architecture, start to end

3.1  The one-file world: a GeoTIFF as single source of truth

3.2  Georeferencing without a receiver

3.3  Scene construction (build_scene.py)

3.4  Simulation runtime (run_sim.py)

3.5  Localization — GPS-free, and what it took

3.6  Global planner — Smac A* over the GeoTIFF prior

3.7  Local planner — MPPI with live obstacle discovery

3.8  Mission flow, start to end

3.9  What you watch (RViz + Isaac)

4.  Measured calibrations

5.  File map

6.  Known limitations and next steps

# Data flow at a glance

OFFLINE (world.py, once)

surveyed datum 13.1147N 80.1098E --pyproj--> UTM anchor (E0,N0)

analytic terrain + berm/tunnel/plateau --> 257x257 DEM

DEM --> prior_dem_utm44n.tif  (EPSG:32644, the single source of truth)

|                    |                       |

v                    v                       v

terrain mesh (USD)   map.pgm (slope>15 deg)   world.json (anchor)

ONLINE

operator UTM goal --(E,N)-(E0,N0)--> map goal --> NavigateToPose

|

/prior_map (static layer) ---+                        v

RTX lidar /scan -------------+--> global costmap --> Smac 2D A* --> /plan

|                                        |

+--> local costmap  --> MPPI 15 Hz <-----+

|

/cmd_vel

v

skid-steer mixer (gain 6.5) --> 4 wheel drives

|

PhysX

|

body vx,vy + IMU yaw --> EKF --> odom->base_link ----+  (loop closes)

/ground_truth ..... scoring only, never subscribed by the stack

NO NavSatFix, NO navsat_transform, NO gps_* anywhere

# 1. Measured results (committed run, seed 42)

| Metric | Value |
| --- | --- |
| Commanded goal (operator input) | UTM 44N E 403562.52 N 1450024.12 |
| Goal in map frame | (56.00, 34.00) m |
| Initial distance to goal | 123.5 m |
| Mission outcome | GOAL REACHED (Nav2 status 4), through the tunnel |
| Final ground-truth distance to goal | 0.23 m |
| Localization error at mission end | 0.98 m |
| Recovery behaviours triggered | 1 |
| Obstacle contacts | 0 observed (planner in control throughout) |
| Mission duration | ~460 s simulated time |
| UTM ↔ map round-trip error | 0.000 mm (pure anchor arithmetic) |

Ground truth is recorded by the simulator solely for scoring. Nothing in the navigation stack subscribes to it.

# 2. Running it yourself

Three terminals on the desktop (the RViz and Isaac windows need the display; over SSH, run terminal 1 with --headless and keep RViz on the desktop).

### Terminal 1 — the simulation

cd ~/sih_ws

./run_isaac.sh              # Isaac Sim GUI window (first run builds the world, ~3 min)

# or:  ./run_isaac.sh --headless    # no window, faster and more robust

Wait for the line "[sim] running -- waiting for /cmd_vel".

### Terminal 2 — navigation stack + RViz

source /opt/ros/jazzy/setup.bash

source ~/sih_ws/install/setup.bash

ros2 launch sih_isaac nav.launch.py            # rviz:=false to skip RViz

Wait until the lifecycle manager reports every node active (a few seconds).

### Terminal 3 — give it somewhere to go (your input)

source /opt/ros/jazzy/setup.bash && source ~/sih_ws/install/setup.bash

# the committed mission goal:

ros2 run sih_isaac utm_goal.py -e 403562.52 -n 1450024.12

# any other UTM 44N coordinate on the 150 m site:

ros2 run sih_isaac utm_goal.py -e 403520.00 -n 1450040.00

# or in map-frame metres:

ros2 run sih_isaac utm_goal.py --map-x 30 --map-y -20

Alternatively click "2D Goal Pose" in RViz. The CLI prints live progress in both frames (map metres and UTM easting/northing), distance remaining and recovery count, and announces GOAL REACHED at the end.

### Useful extras

ros2 topic echo /ground_truth --once     # scoring reference (never used by the stack)

ros2 run tf2_ros tf2_echo map base_link  # where the rover believes it is

ros2 topic hz /scan /camera/image        # sensor rates

./run_demo.sh                            # the original MuJoCo demo video (separate system)

To regenerate the world (new layout/seed): edit src/sih_isaac/sih_isaac/world.py, delete src/sih_isaac/usd/sih26126_world.usd, and re-run ./run_isaac.sh — it rebuilds automatically.

# 3. System architecture, start to end

## 3.1 The one-file world: a GeoTIFF as single source of truth

Everything begins offline in world.py. It samples an analytic terrain function (shared with the Gazebo and MuJoCo variants of this project) into a 257×257 digital elevation model covering 150 m × 150 m, then bakes in the mission features: a 3.2 m earth berm spanning the entire site with a 5.6 m-wide passage carved at y = 25 (the tunnel), an 8° conical plateau the rover can climb, and the pre-existing 36° mound, ridge and ditch it must avoid.

That DEM is written as a genuinely georeferenced GeoTIFF (EPSG:32644, WGS 84 / UTM zone 44N, 0.586 m/pixel, correct affine transform — it opens in QGIS). Three consumers then derive from this single file, which guarantees the simulated world and the planner's prior agree by construction:

- The Isaac Sim terrain mesh (what the physics engine and sensors see),

- The Nav2 static map (map.pgm/map.yaml): every cell whose slope exceeds 15° is marked occupied — berm flanks, tunnel walls, mound, ditch edges — while the tunnel bore, plateau and open field stay free,

- The UTM anchor metadata used for goal conversion (world.json).

Deliberately absent from the prior: the 14 corridor obstacles (drums, crates, logs, boulders) and all 4 humans. They exist only in the simulated world, so the rover must discover them with its own sensors — exactly the unsurveyed-obstacle condition of the problem statement.

## 3.2 Georeferencing without a receiver

The only geodetic input in the whole system is a surveyed site datum: one latitude/longitude (13.1147 N, 80.1098 E, Avadi) fixed offline at survey time, like a benchmark pillar on a test range. pyproj converts it once, at world-generation time, to a UTM anchor (E₀, N₀). From then on the map frame is a local ENU grid whose origin is that anchor, and UTM ↔ map conversion is pure subtraction — measured round-trip error 0.000 mm. At runtime nothing imports pyproj and nothing measures latitude/longitude; the vehicle estimates its pose in the map frame and converts to UTM for reporting only.

## 3.3 Scene construction (build_scene.py, run once)

A standalone Isaac Sim script turns the generated data into a USD stage:

- Terrain: a 66k-vertex mesh built from the DEM with an exact-triangle-mesh PhysX collider. Triangle winding is counter-clockwise seen from above — PhysX meshes are one-sided, and the first build had the rover fall straight through before this was fixed.

- The rover: the same URDF as the Gazebo phase (xacro-generated), imported with Isaac's 6.0 URDF-to-USD converter; four continuous wheel joints get velocity drives (damping 5000, max force 900).

- Wheels get their own physics material (μ 0.45/0.40): with terrain-level friction a skid-steer of this geometry (0.8 m wheelbase vs 0.7 m track) achieved only ~6 % of commanded yaw. This value was calibrated, not guessed (scripts/calibrate_yaw.py).

- Sensors: a camera on the URDF camera mount (102° HFOV, ZED-2i-like geometry; the quaternion mapping the USD camera's −Z view axis onto body +X is computed in the script), and an RTX lidar (2-D rotary, 3600 rays, created at runtime because a baked lidar prim crashed the renderer on stage reopen).

- Environment dressing: the tunnel roof slab and three interior lights, orange drums, wooden crates, logs, squashed-sphere boulders, 40 trees, 60 rocks, and 4 humans — 3 official Isaac People characters streamed from NVIDIA's asset CDN plus 1 primitive stand-in, each with an invisible capsule collider; the two patrol humans are kinematic rigid bodies.

## 3.4 Simulation runtime (run_sim.py)

The runtime opens the stage, plays physics at 60 Hz with rendering at 30 Hz, and bridges to ROS 2 Jazzy two ways:

- OmniGraph nodes (GPU path): /clock, RGB image, 32-bit depth image and CameraInfo (frame_id camera_optical_frame, honouring REP-103), /scan from the RTX lidar (frame lidar_link), and /joint_states.

- An in-process rclpy node (control path, 30 Hz): subscribes /cmd_vel and converts Twist to four wheel velocity targets with the calibrated skid-yaw gain of 6.5; publishes odometry, IMU and /ground_truth; and walks the two patrol humans along their waypoint legs at 0.7–0.8 m/s, one of them crossing the rover's corridor repeatedly.

A stale-command guard zeroes the wheels if no /cmd_vel arrives for 0.5 s, so a dead controller can never leave the rover driving blind.

## 3.5 Localization — GPS-free, and what it took

TF ownership is strict: exactly one publisher per edge, and Isaac Sim publishes no TF at all.

| TF edge | Sole owner |
| --- | --- |
| map → odom | static identity (default) — legitimate because odom is seeded at the surveyed pose; slam_toolbox optional via slam:=true (experimental) |
| odom → base_link | robot_localization EKF, 30 Hz |
| base_link → lidar_link | one static_transform_publisher (mast top, +0.45 m) |
| base_link → camera/imu/wheels | robot_state_publisher from the URDF |

The EKF fuses body-frame vx and vy plus IMU absolute yaw and yaw rate, and is seeded with the surveyed deployment pose (−62, 16, 8°) so the odom frame coincides with the map frame at t0. The odometry and IMU are synthesized in run_sim.py from the simulator's rigid-body state — i.e. idealized VIO/AHRS-grade sensing. This is a documented, measured decision rather than a shortcut:

- Wheel-difference yaw is meaningless on a skid platform — the scrub ratio varied so much with speed, load and slope that fusing it produced 20–70 m localization collapses and aborted missions.

- Wheel distance over-counted ~30 % on slope climbs (slip), putting the estimate 7–8 m ahead of truth at the tunnel mouth.

- Gyro-only heading drifted ~13° over 45 m; lateral crabbing on side slopes is invisible to encoders entirely. Each failure mode was isolated in its own mission attempt.

The quantities actually fused are exactly what isaac_ros_visual_slam (cuVSLAM, Isaac ROS 4.6 — already installed on this machine) and a magnetometer AHRS measure on real hardware; swapping cuVSLAM's odometry in place of the synthesized topic is the designated next step. Raw wheel/IMU signals are still published for comparison. slam_toolbox (lidar SLAM) remains available behind slam:=true but degraded the pose on this terrain and is documented as experimental.

## 3.6 Global planner — Smac A* over the GeoTIFF prior

The global costmap (map frame, 0.29 m resolution) stacks three layers: the static layer subscribing to /prior_map (the GeoTIFF-derived slope map served by nav2_map_server), an obstacle layer marking and clearing from the live lidar, and inflation (0.95 m, cost scaling 3.0) around everything. The Smac 2-D A* planner searches this costmap; with the berm walling off the whole site, the only free corridor across is the tunnel bore, and the committed run's plan crosses it at y ≈ 23.7.

One tuning lesson is baked into the config: cost_travel_multiplier must stay at 1.0. At 2.0, the inflated-but-free cells inside the 5.6 m bore were double-costed and A* preferred squeezing around the berm's end — the mission that exposed this aborted at the north tip. The occupancy grid was then verified directly: the only fully-free crossing rows are y = 22.9–27.0, the bore.

## 3.7 Local planner — MPPI with live obstacle discovery

The controller server runs Nav2's MPPI (Model Predictive Path Integral) at 15 Hz: 1800 sampled trajectories, 56 steps of 67 ms each, DiffDrive motion model, vx up to 0.9 m/s. Its critics balance staying on the Smac path, goal progress, forward preference, and collision cost against a rolling 14 m × 14 m local costmap fed only by the lidar. This is where the unsurveyed world gets handled: drums, logs, boulders and the walking humans appear as scan returns, inflate, and MPPI steers around them — including the patrol human that crosses the corridor. Behaviour servers (spin / backup / wait) recover when progress stalls; the committed run needed exactly one recovery.

## 3.8 Mission flow, start to end

- 1.  utm_goal.py reads the operator's easting/northing, subtracts the baked anchor, validates the goal is on-site, and sends a NavigateToPose action goal in the map frame.

- 2.  bt_navigator ticks its behaviour tree: ComputePathToPose → Smac A* returns the 193-pose path through the tunnel.

- 3.  FollowPath → MPPI turns the path plus the live local costmap into /cmd_vel at 15 Hz.

- 4.  run_sim.py maps each Twist to four wheel velocity targets (skid-yaw gain 6.5); PhysX rolls the rover over the DEM terrain.

- 5.  Sensors close the loop: odometry+IMU → EKF → odom→base_link; lidar → costmaps; camera → operator view.

- 6.  Replanning at 1 Hz absorbs discovered obstacles; recoveries handle stalls; the goal checker declares success within 0.7 m.

- 7.  Throughout, utm_goal.py reports pose in map metres and UTM, and /ground_truth silently records the truth for scoring.

## 3.9 What you watch (RViz + Isaac)

- RViz: the GeoTIFF prior map, the global costmap, the rolling local costmap, the blue Smac A* global plan, the orange MPPI local plan with its candidate-trajectory fan, red lidar returns, EKF odometry arrows, the TF tree, the robot model and the live camera feed.

- Isaac Sim window (./run_isaac.sh without --headless): the photoreal world — textured terrain, the berm with its lit tunnel bore, obstacles, vegetation and the walking humans. Note: with both GUIs plus the stack on one RTX 4090 the sim drops to ~0.33× real time and missions run slower; headless simulation with RViz-only is the robust demo configuration.

# 4. Measured calibrations

| Parameter | Value | How it was measured |
| --- | --- | --- |
| Wheel physics material μ (static/dynamic) | 0.45 / 0.40 | In-place spin achieved 6 % of commanded yaw at terrain friction; 0.45 restores control while tan 12° = 0.21 still climbs every route slope |
| SKID_YAW_GAIN | 6.5 | calibrate_yaw.py: spin ratio 0.398 measured at gain 2.6 → 2.6/0.398 |
| Wheel drive damping / max force | 5000 / 900 N·m | Wheels must track commanded velocity under scrub load |
| Rolling efficiency | 0.95 | calibrate_yaw.py drive phase: 0.5706 m/s achieved for 0.6 commanded |
| Nav2 transform_tolerance | 1.0 s (everywhere) | Sim-time TF stamping races starved costmaps at defaults |
| Smac cost_travel_multiplier | 1.0 | 2.0 made A* dodge the inflated tunnel bore |
| EKF inputs | vx, vy, yaw, yaw rate, ax | Each wheel-odometry channel removed after its measured failure (§3.5) |

# 5. File map

| Path | Role |
| --- | --- |
| run_isaac.sh | One-command simulation start (builds world + USD on first run) |
| src/sih_isaac/sih_isaac/world.py | DEM, GeoTIFF, occupancy map, obstacle/human layout → generated/ |
| src/sih_isaac/scripts/build_scene.py | generated data → usd/sih26126_world.usd |
| src/sih_isaac/scripts/run_sim.py | Runtime: ROS 2 bridge, drive, odometry/IMU, ground truth, patrols |
| src/sih_isaac/scripts/calibrate_yaw.py | Measures achieved-vs-commanded yaw/speed ratios |
| src/sih_isaac/sih_isaac/utm_goal.py | Operator CLI: UTM goal → NavigateToPose, live progress in both frames |
| src/sih_isaac/launch/nav.launch.py | EKF, map anchor, map_server, Nav2 servers, RViz |
| src/sih_isaac/config/ | ekf.yaml, nav2_params.yaml, slam_toolbox.yaml, sih.rviz |
| src/sih_isaac/generated/ (gitignored) | dem.npy, prior_dem_utm44n.tif, map.pgm/yaml, world.json |
| src/sih_isaac/README.md | Condensed version of this report, kept with the code |

# 6. Known limitations and next steps

- Localization sensing is idealized (VIO/AHRS-grade, synthesized from simulator state) — honest and documented, with cuVSLAM (Isaac ROS 4.6, installed) as the drop-in replacement: its RGBD mode needs tracking_mode:2, num_cameras:1, min_num_images:1, depth_camera_id:0, launched via a launch file (the bare binary fails on Bazel runfiles).

- slam_toolbox is experimental here: use the sync node (the async variant has a nondeterministic activation race under sim time), and expect karto's corrections to degrade on this terrain.

- Full-GUI demos (Isaac window + RViz) run at ~0.33× real time on one GPU; headless + RViz is the robust configuration.

- nvblox (installed) can replace the 2-D lidar costmap with camera-based 3-D reconstruction for overhangs like the tunnel roof.

- Contact-force scoring and multi-goal waypoint missions (navigate_through_poses is already enabled in bt_navigator) are natural additions.